Product Requirements Document (PRD) – Phase 1

Project Name: Verofy HLD Auto Upload & QC Tool
Author: Alexander Hall
Date: 2025-12-2
Version: 1.0

NOTE: verofy_api & oldqc folders will be used for phase 2. Do not evaluate these folders in the folder structure for this phase.

1 – Front End Step 1:

- Javascript web based

- Base already built (inside this folder structure, but currently pointing to a now-defunct golang file, please revise)

- Once user has provided .zip file, send to back-end for QC

2 – Back End Step 1:

- Python backend (must be built from scratch)

- First, unzip file and dump into a temp folder

- Next, run QC on shapefiles in temp folder

3 – Back End Step 2 (QC):

- Use “samplefiles” folder in this folder structure for specs on properly attributed shapefiles for all dictation below. QC steps start below this.

- Ensure zip contains 10 full shapefiles named:

  - poles

  - network_elements

  - splicing

  - sites

  - parcels

  - permits

  - cabinet_boundaries

  - segments

  - access_points

  - cables

- Ensure they are all in WGS 84 projection

- Ensure all shapefiles have a “UID” field, populated with an integer that is unique to each feature (an index), so if there are QC issues the specific feature can be called out and returned to user

- Ensure correct attribute fields exist in all shapefiles with with appropriate data

  - Segments:

    - Type = Aerial, 3rd Party Duct, Underground, Existing VERO, Drop Cable

    - Group 01 = Zone XX (XX = 2 digit #)

    - Conduit = (X)-1.25” SDR 13.5 HDPE. X = 3 or 1. ONLY if Type = Underground. Nothing needed here for any other type.

  - Access Points

    - Type = Handhole, Cabinet

    - Group 01 = Zone XX (XX = 2 digit #)

    - Latitude. Must be a #

    - Longitude. Must be a #

  - Cabinet Boundaries

    - Name = Zone XX Boundary (XX = 2 digit #)

  - Permits

    - Name field starts with either: “ROW”, ROE”, or “LLP”

  - Cables

    - Name = Begins with XXXF. (XXX = 3 digit #)

  - Parcels

    - Name = “Parcel” (THIS MUST MATCH EXACTLY FOR ALL FEATURES)

    - Group 01 = Zone XX (XX = 2 digit #)

  - Sites

    - Type = School, Hub Site, MDU, Administration, MTU, Dwelling Unit, Vendor Location, Cell Tower, Government, Data Center, Hosptial, Internet, Large Business, Library, Museum, Power Substation, Small Business, Small Cell, Stadium, University, Splice Point, ILA, SFR, Vacant Lot, Mobile Home, Meet Me

    - Address = must be populated, can be anything

    - State = YY (YY = 2 LETTERS)

    - Zip = XXXXX (XXXXX = 5 #s)

    - BEN# = Must be an integer

    - Latitude. Must be a #

    - Longitude. Must be a #

  - Splicing

    - AKA = Must begin with YYY_Y (Y = Letter)

    - Type = MST, Splice, FTP

    - Group 01 = Zone XX (XX = 2 digit #)

    - Latitude. Must be a #

    - Longitude. Must be a #

  - Network Elements

    - Type = Slack Coil, Anchor, Bore Pit, Riser

    - Group 01 = Zone XX (XX = 2 digit #)

    - Latitude. Must be a #

    - Longitude. Must be a #

  - Poles

    - Pole Tag = Must be populated with something/anything

    - Pole Owner = Must be populated with something

    - Group 01 = Zone XX (XX = 2 digit #)

    - Latitude. Must be a #

    - Longitude. Must be a #

4 – Front End Step 2:

- IF shapefiles pass back end QC:

  - Have a pop-up message say “Your files have passed QC! Please provide VerofyMapID:”. Have a box for the user to type in a number.

- IF shapefiles do not pass back end QC:

  - Have the website send the user a .txt file delineating each feature that failed. If more than 10 features failed, simply put 1 line for each shapefile saying “10 or more features failed XXX”, XXX being whatever the specific QC issue is. If a shapefile fails multiple separate QC issues, list each QC item separately for the same shapefile.